Educator and Tagging Information

	Learning Area:

Technology

	Resource Name:  

Technology 

	Assessment Exemplar Number:

TECHN8.147

	Item/s:

1

	Phase: 

Senior Phase 

	Grade:

8

	Tags:

Technology, mechanical systems, pulleys, speed advantage, SA, size, belt drive, rotate, distance, effort, load, input distance, output distance, revolution, driver pulley, driven pulley, speed ratio, calculate, worksheet, Formative Assessment

	Assessment Type:

Formative

	Assessment Form/s:

Worksheet

	Copyright for included material:

N/A 

	Duration:
60 min

	Learning Outcome(s) and Assessment Standard(s):

Learning Outcome 2:  Technological Knowledge and Understanding
The learner will be able to understand and apply relevant technological knowledge ethically and responsibly.
Assessment Standards
We know this when the learner

3 Systems and Control
3.1 Demonstrates knowledge and understanding of how mechanical systems (e.g. pneumatic or hydraulic systems, gears, belt drive systems, pulley systems, linked lever systems) convert motion and force to give mechanical advantage, and represents them using systems diagrams.

	Learning Space:  

Assessment 

	Hyperlinks: 

To be completed later. 

	Rating:

Number of questions for exemplar:

1

	Easy questions:



	Medium questions:

Question 1

	Difficult questions:



Assessment Task 

Pulleys provide a speed advantage

The size of the pulleys in a belt drive affects the speed at which they rotate.  This is described as distance moved by effort, divided by distance moved by load. 
In other words:
Input distance

Output distance

For example, if, for one revolution of the driver pulley, the driven pulley revolves three times, we say the speed ratio is 1:3.

1.1
Calculate the following outputs of the driven pulley if the speed ratio is 3:1.

[4 marks]
1.1.1
Belt drive A:  Input 300 revolutions                 Output ………………..revolutions.

1.1.2
Belt drive B:  Input 75 revolutions                   Output ………………..revolutions
1.2
Complete the following for a speed ratio of 3:1.




[6 marks]
1.2.1
Belt drive C:  Input 330 revolutions
        
Output……………….revolutions

1.2.2
Belt drive D:  Input 150 revolutions

Output……………….revolutions

1.2.3
Belt drive E:  Input 210 revolutions       
Output ………………revolutions
1.3
Calculate the speed ratios for the pulley systems below.

In each case, A is the driver pulley.  The groove diameters of the pulleys are shown in cm.

	Driver pulley A
	Driven pulley B
	Speed ratio

	7
	21
	

	24
	8
	

	15
	30
	

	36
	4
	

	81
	9
	


                                                                                                                 


[10 marks]

 [20 marks]

Suggested Solutions 

	Question number
	Possible marks
	Answers

	1
	2 marks

2 marks

2 marks

2 marks

2 marks

5 x 2 marks
	1.1.1 Belt drive A   Output 900 revs

1.1.2 Belt drive B   Output 225 revs

1.2.1 Belt drive C   Output 990  revs

1.2.2 Belt drive D   Output  450 revs

1.2.3 Belt drive E   Output  630 revs

1.3

Driver pulley A

Driven pulley B

Speed ratio

7

21

3 :1

24

 8

1 : 3

15

30

2 : 1

36

4

1 : 9

81

9

1 :9




[20 marks]

Appendix of Assessment Tools



















































